# Countess II FL Automated Cell Counter

- Turn on the Countess using the power switch in the back right of the instrument

- Pipette 10 µL of cell suspension to either or both sides of a disposable countess slide via the opening

- Insert the slide into the slide port on the Countess until it clicks into place

- Optional: The instrument will auto-focus. You can manually adjust the focus by pressing the Focus button and adjusting the slider.

- Select the light source and capture channels using the menu on the right. Options are Brightfield (BF), Texas Red (TxRed), and GFP (green fluorescent protein).

- Press Capture to get the total concentration of cells, percentage of live cells, and percentage of dead cells. Live cells will be circled in green and dead cells in red.

- Optional: Plot the cell size distribution with the Graph button.

- Optional: Save the results to a USB drive by clicking Save.

- Eject the slide by pushing inwards until it clicks and the spring ejects it

- Optional: Press New to start another assay with the reverse side or another slide.

For more information see the full instruction manual at https://www.thermofisher.com/content/dam/LifeTech/global/life-sciences/cellanalysis/Files/0914/MAN0010644_Countess%20II.pdf